QUICK INSTALLATION FOR IBM SRD WITH ACCENT

ACCENT-SA

Follow IBM SRD installation instructions.

ACCENT-MC

HARDWARE INSTALLATION:  Simply slide the card into the desired

expansion slot of your PS/2, making sure it fits properly. Tighten

the lock-screw on the back panel of your PS/2 to secure Accent-MC

in the slot.

SYSTEM CONFIGURATION:  Use Accent-MC's Adapter Descriptive File,

named @6152.ADF, to reconfigure your PS/2.  This file is located in

the Accent Demo/Utility diskette.

SOFTWARE INSTALLATION:  From the Accent Demo/Utility diskette, copy

the SPKACNT.DVC file to your root directory.  Then copy the SPKACNT.SYS

file to your CONFIG.SYS file, if there is no existing CONFIG.SYS file,

or if there already is an existing CONFIG.SYS file, add (edit) the following

statement to your CONFIG.SYS:      DEVICE=SPKACNT.DVC

SRD INSTALLATION:  Follow the SRD installation instructions for

using Accent-SA as its synthesizer.  Then copy the file

SRDASYNC.EXE from the Accent Demo/Utility diskette to your

installed directory for the SRD.  Note: This will overwrite your

installed SRDASYNC.EXE.

The SRDASYNC.EXE program provided in the Accent Demo/Utility

diskette is a modified version of the original IBM SRDASYNC.EXE; it

redirects the COM Port's I/O to the LPT Port for interface with

internal Accents.

ACCENT-PC

Because the default address (2A8 Hex) of Accent-PC is in conflict

with the SRD Adaptor Card, Accent's address should be changed to

one of the following four alternatives or other addresses not in

conflict.  Next, follow the installation instructions for Accent-PC

described in your Accent User's Manual.  Note:  Both the Device

Driver and the hardware card must be changed to the same address.

To change the Device Driver with the Accent Utility, type "SETACNT

SPKACNT.DVC" and follow the menu.  To change the hardware address,

set SW2 to one of the following settings or refer to your Accent

User's Manual.

I/O ADDRESS; ACCENT-PC's SW2 SETTINGS

SW2 POS   1       2       3       4       5       6       7      8

2ECH     OFF     OFF     ON      OFF     OFF     OFF     ON     OFF

3ECH     OFF     OFF     ON      OFF     OFF     OFF     OFF    OFF

300H     ON      ON      ON      ON      ON      ON      OFF    OFF

308H     ON      OFF     ON      ON      ON      ON      OFF    OFF

Then follow the same SRD INSTALLATION procedures as for Accent-MC above.

ACCENT-mini

Follow the installation instructions for Accent-mini described in

your Accent User's Manual.  Then follow the same SRD INSTALLATION

procedures as for Accent-MC above.

ACCENT-L40

SOFTWARE INSTALLATION:  Follow the installation instructions for Accent-mini

described in QUICK INSTALLATION and your Accent User's Manual.  Then follow

the same SRD INSTALLATION procedures as for Accent-MC above.

HARDWARE INSTALLATION:  Turn off power to the computer and unscrew the

keyboard and top cover.  Second, plug the connector on Accent-L40 into the

corresponding 50 pin female connector on the IBM PS/2 L40SX's mother board,

and securely tighten it to the mother board with the original screw.  Third,

run a power test for Accent-L40 by plugging the L40SX's AC adaptor in to

ensure power to the computer, and turn on the computer; Accent-L40 should

say, "Accent Ready." through the L40SX's system speaker after the device driver

has been loaded.  Lastly, turn off power to the computer and reassemble.